# Design, Simulation, Testing, and Hardware Validation of an LTC1871-Based DC-DC Boost Converter for Battery-Powered Subsystems for Robotic & Autonomous Devices.

LTC1871-Based DC-DC Boost Converters

# Fortune Olose

# Final Year Project
B.E. Electronic Engineering with Computers

# A thesis submitted in partial fulfilment of the requirements for the Maynooth University B.E. in Electronic Engineering

# May 2026

# Supervisor: Dr. Bob Lawlor

Table of Contents

Abstract	iv

Declaration	v

1	Introduction	1

1.1	Problem Statement and Scope	1

1.2	Approach and Methodology	2

1.3	Evaluation Strategy	2

1.4	Broader Impact	3

1.5	Original Contributions and Externally Provided Resources	3

1.6	Structure of the Report	4

2	Technical Background	4

2.1	DC-DC Converter Fundamentals	5

2.2	Boost Converter Operating Principle	6

2.2.1	Circuit Topology and Switching Intervals	6

2.2.2	Voltage Transfer Ratio from Volt-Second Balance	7

2.2.3	Inductor Current Waveform and Ripple	7

2.2.4	Inductor Current RMS Value	8

2.2.5	Output Capacitor Voltage Ripple – General Principles	9

2.2.6	Input Capacitor Ripple Current	9

2.3	Current-Mode Control and the LTC1871 Architecture	10

2.4	Feedback and Loop Compensation	11

2.4.1	Feedback Divider and Reference Scaling	11

2.4.2	Loop Gain, Crossover Frequency, and Stability Margins	12

2.4.3	Type II Compensation Network	12

2.4.4	Advantages for this Application	13

2.5	Input Protection Topologies	13

2.5.1	Reverse Polarity Protection	13

2.5.2	Undervoltage Lockout (UVLO)	14

2.5.3	Overvoltage Lockout (OVLO)	14

2.5.4	Combined Voltage Window Protection	14

2.6	PCB Design Considerations for Power Converters	14

3	Design Methodology	15

3.1	Design Specifications and Methodology	16

3.2	Power Stage Component Selection	17

3.2.1	Switching Frequency and Duty-Cycle Range	17

3.2.2	Inductor Selection	17

3.2.3	Power MOSFET Selection	18

3.2.4	Schottky Diode Selection	19

3.2.5	Input and Output Capacitor Selection	19

3.3	Feedback Divider and Loop Compensation Network Design	21

3.3.1	Output Voltage Feedback Divider	21

3.3.2	Type II Compensation Network Design	21

3.3.3	Verification of Expected Loop Performance	22

3.4	Input Protection Circuit Implementation	23

3.4.1	Reverse Polarity Protection	23

3.4.2	Undervoltage Lockout (UVLO)	23

3.4.3	Overvoltage Lockout (OVLO)	23

3.4.4	Combined Voltage Window Protection	24

3.5	Schematic Capture, PCB Layout, and Prototype Realisation	24

3.5.1	Schematic Capture in KiCad	24

3.5.2	PCB Layout Strategy	24

3.5.3	Fabrication and Assembly	27

3.6	Summary of Design Decisions	27

4	Simulation Methodology	28

4.1	Simulation Environment and Solver Configuration	28

4.2	SPICE Models and Component Representation	29

4.3	Test Circuit Configuration	30

4.4	Parametric Sweep Strategy	31

4.5	Measurement Definitions	32

4.6	Loop Stability Verification (Analytical)	33

Glossary of terms [OPTIONAL]	34

Nomenclature [OPTIONAL]	34

References	34

Appendix A [OPTIONAL]	34

Risk Assessment	34

Abstract

This report presents the design, simulation, verification, and hardware implementation of a DC-to-DC boost converter for battery-powered robotic & autonomous devices. The converter, based on the Linear Technology LTC1871 current mode controller, accepts a three-cell lithium polymer battery input ranging from 8 – 11V and regulates a stable 12V output at up to 2A continuous load current. The project followed a structured methodology beginning with analytical power stage design, during which an initial converter topology was abandoned due to simulation convergence failures and replaced with a boost topology confirmed by sufficiency analysis. A comprehensive simulation campaign validated all performance metrics with substantial margin across the full input range, including an output voltage of 12.20V, output ripple of 171 mV, load transient droop of 123mV under a 1.5A inrush step, and a simulated efficiency of 89.84% at the upper input boundary. A three-layer input protection scheme was developed comprising a series P-channel transistor for reverse polarity blocking, a resistive divider for undervoltage lockout, and a Zener-transistor clamp for overvoltage lockout, together forming a voltage window that restricts active regulation to a validated operating band of 8.0 to 11.1 V. Iterative tuning of the divider ratio and clamp network was required to counteract parasitic loading effects that initially compressed the operating window, with the final design achieving precise alignment with the target battery discharge range. The validated schematic was translated into a compact two-layer printed circuit board measuring 60.5 by 42.5 millimetres using an open-source layout tool, with fabrication files generated for external manufacture and final assembly carried out by hand-soldering in the university electronics laboratory.

Declaration

I hereby certify that this report and the project that it describes is my original work and has not been taken from the work of others save and to the extent that such work has been cited and acknowledged in the report. I would like to thank my supervisor, Dr. Bob Lawlor, for his guidance and feedback throughout the project. I also wish to thank Mr. John Maloco, Electronics Laboratory Technician, for his assistance with component procurement and hand-soldering of the printed circuit board. I confirm that any use of Generative AI tools in the preparation of this report complies with Maynooth University’s guidelines. Where GenAI tools have been used to support my work, I have acknowledged this appropriately. A list of prompts is provided in the appendix.

Signed:

Fortune Olose

BEng Electronic Engineering

Maynooth University

April 2026

# Introduction

Mobile robotics platforms depend on compact battery packs whose terminal voltage inevitably sags as the cells discharge, a 3-cell lithium-polymer pack, for instance, falls from 12.6V fully charged to below 9V at end-of-discharge, easily dropping below the voltage required to keep downstream point-of-load regulators in regulation [1,2]. All three payload devices in this project, the Raspberry Pi Compute Module 4, the RPLiDAR A1 laser scanner, and the RFD900x telemetry modem, are fundamentally 5V loads with tight input tolerances. The RPLiDAR's core scanner accepts only 4.9-5.5V and is damaged above 5.5V, while the RFD900x specifies a +5V nominal supply with an absolute maximum of +6V. [3, 4] Neither device can be powered directly from a fluctuating battery rail. Instead, the system employs a cascaded power architecture: a tightly regulated 12V intermediate bus first is generated from the battery, and each subsystem is then supplied by a dedicated point-of-load step-down converter that delivers a precise 5V. If the 12V bus sags below the dropout threshold of these downstream converters, their output collapses and the loads brown out, regardless of remaining battery capacity. A stable 12V backbone, maintained across the entire discharge curve, therefore guarantees the headroom needed for the 5V conversion stages and ensures continued operation of the LiDAR, telemetry link, and processor throughout an autonomous mission [5, 6].

## Problem Statement and Scope

The specific engineering problem addressed in this thesis is the design and implementation of a regulated intermediate power bus for the payload of a proof-of-concept autonomous ground vehicle. The bus is generated from a 3-cell lithium-polymer battery whose terminal voltage during operation is intentionally restricted to an 8–11V window, below the 12V target, so that a boost-only topology can be employed. The converter is built around the LTC1871 current-mode boost controller, which steps up the battery voltage to a tightly regulated 12V rail while supplying the three 5V payload devices (Raspberry Pi Compute Module 4, RPLiDAR A1, RFD900x modem) through their downstream point-of-load regulators [7, 8]. The converter is rated to deliver up to 24W at the full 2A continuous load, providing sufficient margin for the combined power draw of the three payload subsystems and their associated point-of-load conversion losses. The scope of this work is confined to the boost converter itself: the design, simulation, and hardware prototyping of the power stage, compensation network, and feedback loop. The downstream 5V regulators are treated as fixed resistive loads, and all navigation, sensing, and communication algorithms lie outside the project boundary. The converter's performance is evaluated against output voltage regulation, efficiency, load transient response, and the ability to maintain a stable 12V rail across the full 8–11V input range, evaluated primarily through LTspice simulation, with hardware prototyping conducted to validate the physical design. By constraining the problem to this single boost stage, the thesis isolates the power-conditioning function that enables the robot's sensitive electronics to survive a complete battery discharge cycle without supply interruption.

## Approach and Methodology

An initial SEPIC design using the TPS61088 controller was abandoned due to persistent LTspice convergence failures [9, 10]. A topology sufficiency analysis confirmed that a simple boost converter could meet the specification because the battery’s 8–11 V operating window lies strictly below the 12 V output. The design was migrated to the LTC1871 current‑mode boost controller, and the power stage was sized through first‑principles calculations: switching frequency, duty‑cycle range, inductor and output capacitor selection (including DC‑bias derating), MOSFET and Schottky diode choices, feedback divider derivation, and Type II compensation network design for loop stability [11].

The analytical design was subjected to a structured LTspice simulation campaign that validated eight performance metrics across the full input range, supplemented by AC analysis for loop stability. The validated design was translated into hardware with a three-layer input protection scheme (reverse polarity blocking via an AO6407 PMOS, undervoltage lockout, and overvoltage lockout). The protection thresholds were iteratively tuned through simulation to yield an 8.0 –11.1 V operating window matching the battery specification [12]. The final schematic was captured in KiCad, laid out as a two-layer PCB, fabricated by JLCPCB, and hand-assembled in the university laboratory [13, 14].

## Evaluation Strategy

The design was validated primarily through LTspice transient and AC simulations, with additional verification provided by a fabricated hardware prototype. Transient simulations were used to evaluate output voltage regulation, ripple, line and load regulation, transient response, peak inductor current, and efficiency across the full input voltage range. AC analysis was performed to confirm loop stability through extraction of phase and gain margins. Input protection functionality was verified by sweeping the input voltage across the expected operating regions. A fabricated PCB confirmed schematic-to-layout translation and manufacturing feasibility. Detailed simulation test conditions, measurement procedures, and results are presented in Chapters 4 and 5. Electrical bench testing, thermal imaging, EMI measurements, and long-term reliability testing were not performed; these are noted as future work. Pre-hardware simulation validation is standard practice in power converter development, as LTspice’s device-level models – including the manufacturer-supplied LTC1871 macro model – capture the dominant electrical behaviour with sufficient fidelity to confirm design feasibility before committing to fabrication.

## Broader Impact

This project provides a regulated 12 V rail from a 3-cell Li-Po battery pack (8.0 –11.1 V) for autonomous ground vehicle applications such as environmental monitoring, agricultural inspection, or search and rescue. The converter achieves 89 % efficiency at full load, dissipating approximately 2.7 W of heat; an equivalent design operating at only 80 % efficiency would waste 6 W, nearly doubling thermal losses. This improvement directly supports SDG 7 (Affordable and Clean Energy) by extending battery life and reducing charging cycles, which lowers net energy consumption over the vehicle's operational lifetime. The higher efficiency translates into a measurable increase in mission duration compared to a conventional 80 %-efficient converter, particularly for subsystems that draw heavily from the 12 V rail. This enables longer-lasting field deployments.

The design also contributes to SDG 9 (Industry, Innovation and Infrastructure) by demonstrating a robust, reproducible DC-DC converter architecture suitable for battery-powered robotic systems, and to SDG 12 (Responsible Consumption and Production) through the use of lead-free, standard discrete components that facilitate recycling and reduce hazardous waste [15–17]. No inherent ethical conflicts are associated with this design, as it is intended for general-purpose power conversion on autonomous platforms.

## Original Contributions and Externally Provided Resources

The following original contributions were made by the author:

- Design and sizing of the power stage (inductor, MOSFET, diode, output capacitors) for a 0.5A to 2A load range.

- Selection and configuration of the compensation network to achieve stable loop response.

- Implementation of input window protection (UVLO/OVLO) using a resistive divider and Zener-NPN clamp circuit.

- Full LTspice simulation framework, including parametric sweeps, transient measurement windows, and Bode analysis.

- PCB layout, component sourcing, and assembly oversight.

Externally provided resources are acknowledged as follows:

- Dr. Lawlor provided supervisory guidance on converter topology selection and review of the feedback loop design.

- John Maloco performed the soldering of surface-mount components on the fabricated PCB.

- Third-party SPICE models for the LTC1871 controller, MOSFETs, and diode were sourced from the manufacturer (Analog Devices) and third-party libraries; their limitations are noted in the simulation methodology.

- JLCPCB fabricated the bare PCB according to the submitted Gerber files.

## Structure of the Report

Chapter 2 reviews the relevant background theory of boost converters, including continuous-conduction mode, feedback compensation, and input protection principles. Chapter 3 presents the design methodology, covering component selection, loop compensation calculations, and protection threshold setting. Chapter 4 describes the simulation setup, parametric test conditions, and the metrics used for validation. Chapter 5 reports the results from transient and AC simulations, including efficiency, ripple, load transient response, and protection verification. Chapter 6 discusses the limitations of the simulation-only validation and the status of the hardware prototype, followed by conclusions and recommendations for future work.

# Technical Background

This chapter establishes the theoretical foundation underpinning the converter design, driven by key specifications: an 8 - 11V input from the 3S Li-Po battery, a regulated 12V output at 2A continuous load, a 350 kHz switching frequency, a current-mode control implemented with the LTC1871 controller. The chapter begins with a review of DC-DC converter fundamentals and the operating principles of a boost converter in continuous conduction mode. From there, it develops the theory behind current-mode control, feedback loop compensation, and input protection topologies, followed by PCB layout considerations for switching converters. The material presented here directly informs the design decisions detailed in Chapter 3.

## DC-DC Converter Fundamentals

A battery delivers a variable, unregulated voltage, yet the load requires a fixed, stable potential. In robotic systems, loads are time-varying: motors draw inrush currents during setup, LiDARs and communication radios exhibit burst-mode power demands, and compute modules require clean rails to avoid erratic behaviour. Poor regulation under these transients can cause brownouts, sensor data corruption, or radio dropouts. Consequently, a regulator must be placed between the source and the load. A linear regulator (e.g., LM7812) can only step down, dissipating the voltage difference as heat; with an 8-11V input and a 12V output, a conventional linear regulator cannot step up, and even in step-down applications its efficiency is limited to VoutVin. In contrast, a switch-mode converter stores energy in an inductor during one part of a switching cycle and releases it to the load during another, achieving typical efficiencies of 85-95%.

Three fundamental topologies exist in continuous conduction mode (CCM). The buck converter produces an output voltage lower than the input, with the transfer function

VoutVin=D

Where D is the duty cycle. The boost converter steps up the voltage, following

VoutVin= 1(1-D)

The basic buck-boost converter can either step up or step down, but it inverts the output polarity:

VoutVin= -D(1-D)

For applications requiring a non-inverting buck-boost capability, the SEPIC (single-ended primary inductance converter) is a practical alternative, while preserving polarity.

Given the specification of an 8-11V input (from a 3S Li-Po battery) and a regulated 12V output, the input voltage always lies below the output voltage. Thus, a simple boost topology is sufficient; the additional flexibility of a buck-boost or SEPIC converter would introduce unnecessary component and control complexity. This choice motivated the detailed analysis of boost converter operation presented in Section 2.2.

## Boost Converter Operating Principle

### Circuit Topology and Switching Intervals

Figure 2.1 shows the canonical boost converter circuit, consisting of four principal elements: an inductor L, a power MOSFET switch Q1, a diode D1, and an output capacitor Cout. The input voltage source Vin (here, a 3S Li-Po battery) supplies energy to the inductor, while the output delivers a regulated higher voltage to the load Rload. The LTC1871 controller drives the MOSFET gate; a current-sense signal is derived from the MOSFET drain-source voltage, eliminating the need for a discrete sense resistor.

Figure 1: Boost converter topology - inductor L, MOSFET Q1, diode D1, output capacitor C_OUT, input voltage V_IN, and load

The converter operates in two distinct intervals per switching cycle. During the on interval (0 < t < Ts), the MOSFET conducts, connecting the inductor-diode junction to ground. The diode becomes reverse-biased, disconnecting the input from the output. The inductor voltage equals Vin, causing it’s current to ramp linearly upward, storing energy. During this interval, the output capacitor solely supplies the load current, and the capacitor voltage decreases slightly.

During the off interval (DTs<t< Ts), the MOSFET turns off. The inductor’s magnetic field collapses. Reversing the polarity across the inductor such that its voltage adds to Vin. The diode becomes forward biased, and the inductor current flows through the diode into the output capacitor and load. The inductor voltage now equals Vin- Vout (negative, since Vout> Vin), so the inductor current ramps downward, transferring the stored energy to the output.

### Voltage Transfer Ratio from Volt-Second Balance

In steady state, the net charge in inductor current over one switching cycle must be zero – the volt-second balance principle. Applying this to the boost converter:

- On interval (0<t<DT): vL=VIN

- Off interval (DT<t<T): vL=VIN-VOUT

Vin .  DTS+Vin- Vout . 1-DTs=0

Expanding and simplifying:

VinD + Vin 1-D - Vout1-D=Vin- Vout1-D=0

Solving for the voltage transfer ratio:

VoutVin= 11-D and D=1- VinVout

For the design, Vout=12V. With Vin=11V (battery near full), the required duty cycle is

D = 1 - 1112 = 0.083 (8.3%).

At the minimum input Vin =8V, D = 1 - 812 = 0.333 (33.3%). All duty cycles remain below 50%, so the LTC1871’s internal slope compensation circuit remains inactive, simplifying loop dynamics.

### Inductor Current Waveform and Ripple

During the on interval, the inductor voltage is Vin. From the inductor voltage-current relationship:

V=Ldidt

Shifting the terms around and make di the subject of the formula:

VL= didt

VLdt=di

During the on interval the inductor voltage is VIN :

ΔIl= VinDfsw L

The average inductor current is derived from power balance, assuming ideal efficiency (Pin= Pout): VinIL, AVG= VOUTIOUT. Substituting VOUTVIN= 1(1-D) gives:

IL, AVG= IOUT1-D

This analytical peak is lower than the LTC1871’s internal current-limit threshold (nominally 3.59A for this design); the inductor’s saturation rating must exceed the higher of the two. The selected inductor (XAL6060-103MEC) has a saturation current rating of 11.2A, providing ample margin.

### Inductor Current RMS Value

The inductor current waveform in CCM consists of a DC average IL, avg with a superimposed triangular ripple ∆IL (peak-to-peak). The RMS (root-mean-square) current is needed to calculate the copper loss in the inductor winding and to verify that the selected inductor does not exceed its thermal current rating. For a triangular ripple on a DC offset, the RMS current is given by:

IL, RMS= IL, avg 2+ ∆IL212

The RMS current is the quantity that determines the inductor’s copper loss: PCu=IL,RMS2⋅DCR, where DCR is the winding DC resistance. The three current metrics therefore serve distinct purposes:

- Average current IL,avg — used in power balance and output current delivery.

- Peak current IL,pk — used to ensure the inductor does not saturate (must be below Isat).

- RMS current IL,RMS — used to ensure the inductor does not overheat (must be below the manufacturer’s RMS current rating).

The selected inductor (XAL6060-103MEC) has an RMS current rating of 8.3 A, providing a large margin over the 3.01 A calculated above.

### Output Capacitor Voltage Ripple – General Principles

During the MOSFET on interval, the output capacitor alone supplies the load current. The charge removed is:

∆Q= IOUTDfsw

Giving a charge-based ripple of:

∆VOUT,   ESR= ∆ILRESR

The total output voltage is the sum of the charge-based and ESR contributions. Furthermore, the output capacitor must be rated to carry the AC ripple without excessive heating. For a boost converter in CCM, the RMS ripple current through the output capacitor is:
IRMS(Cout)= IOUTVOUT- VINVIN

Finally, for ceramic (MLCC) capacitors, the effective capacitance decreases under DC bias. Manufacturer derating curves must be consulted; the design should oversize the nominal capacitance accordingly. The detailed selection and numerical verification are presented in Chapter 3.

### Input Capacitor Ripple Current

The input capacitor handles the AC component of the inductor current. In a boost converter, the input current is continuous, and only the ripple portion flows through Cin. The RMS ripple current is:

IRMS(Cin)=ΔIL12

The input voltage ripple has a capacitive component and an ESR component.

ΔVin=ΔIL8fswCin+ΔIL⋅RESR,Cin

Excessive input ripple can degrade the accuracy of the LTC1871’s RDS(on)-based current sensing and may perturb the UVLO divider threshold. Therefore, adequate input capacitance (typically a low-ESR ceramic placed directly at the IC’s VIN pin) is required. The specific capacitance value, derating considerations, and verification of negligible ripple are provided in Chapter 3.

## Current-Mode Control and the LTC1871 Architecture

Two fundamental approaches exist for controlling a switch-mode converter: voltage-mode control and current-mode control. In voltage-mode control, the error amplifier directly sets the duty cycle, and the modulator operates as a voltage-controlled pulse -width modulator. The power stage then exhibits a second-order response (the double pole of the LC filter), requiring a Type III compensator with multiple poles and zeros to achieve stable operation over the full input and load range. [18]

Current-mode control, by contrast, introduces an inner current loop. The outer voltage error amplifier sets a peak-current threshold, and the inner loop terminates each switching cycle when the inductor current reaches that threshold. This converts the inductor from a voltage-driven element (which contributes a second-order response) into a controlled current source (first-order, single-pole response). The resulting plant behaves approximately as a single pole at the output capacitor’s load resistance, dramatically simplifying compensation. The LTC1871 controller implements current-mode control using the MOSFET’s on-resistance for sensing, as described in its datasheet. [19]

The architecture consists of two nested feedback loops. The inner current loop operates cycle-by-cycle at the switching frequency (350kHz). The outer voltage loop operates through the compensation network connected to the ITH pin, with a crossover frequency of 4.91kHz (as designed in Section 4.8). The ITH pin serves as the interface between the two loops: the outer loop sets the ITH voltage, which the inner loop uses as a peak-current reference. The inner loop responds to input voltage perturbations within one switching cycle (2.86μs), before the output capacitors can charge or discharge meaningfully, a mechanism that produces the line regulation of 2.9 μV/V measured in simulation (Section 6.2)

The LTC1871 eliminates the need for a discrete current-sense resistor by using the MOSFET’s drain, and the internal current-sense comparator measures the voltage drop across RDS(on) during the on-interval. This lossless sensing maximises efficiency but introduces a temperature dependency because RDS(on) increases with junction temperature. For this design, the duty cycle range is 8.3%to 33.3%, and the selected MOSFET (IRF7811) has a typical RDS(on) of 12mΩ. The resulting sense voltage remains well within the LTC1871’s current-limit threshold across the expected temperature range, so no external compensation for temperature is required.

Slope compensation is a standard addition to current-mode controllers to prevent subharmonic oscillation at duty cycles above 50%. The LTC1871 includes internal slope compensation that activates automatically when the duty cycle exceeds 50%. Because the present design operates entirely below 50% duty cycle (  DMAX=33.3% ), the internal slope compensation remains inactive. This simplifies the loop dynamics and further reduces the component count; it is a direct benefit of the boost-only topology section (Chapter 5).

For this battery-powered robotic application, current mode control offers several advantages over voltage-mode control. First, the power stage becomes first-order, requiring only a Type II compensator (one zero, one pole) instead of a Type III. Second, the peak-current limit is set by the voltage at ITH, providing cycle-by-cycle current limiting without additional components. Third, input voltage variations (as the Li-Po discharges) are rejected within one switching cycle, producing the measured line regulation of 2.9µV/V. Fourth, the indicator no longer appears as a second-order element in the control-to-output transfer function, reducing phase lag and easing stabilisation. These characteristics make the LTC1871 particularly well-suited to the variable input voltage and transient load demands of mobile robotic systems. The theoretical treatment of current-mode control can be found in standard power electronics texts [18], while the LTC1871’s specific implementation details are covered in this datasheet. [19].

## Feedback and Loop Compensation

The purpose of the feedback loop is to maintain tight regulation of the output voltage despite variations in input voltage and load current. In an open-loop converter, the output voltage would vary directly with changes in input voltage and duty cycle. Closed loop operation continuously compares the scaled output voltage to a precise internal reference and adjusts the duty cycle accordingly. The effectiveness of this correction is quantified by the loop gain T(s), the product of the gains around the feedback path. Loop gain determines both steady-state accuracy (via its low-frequency magnitude) and dynamic performance (via its crossover frequency and phase margin). The LTC1871 implements current-mode control, which simplifies the power-stage transfer function to approximately a single pole. This characteristic makes the converter well-suited to compensation with a Type II network [18, 19].

### Feedback Divider and Reference Scaling

The output voltage is sensed by a resistive divider connected to the FB pin of the LTC1871. The internal reference voltage is VREF=1.230 V. In steady state, the error amplifier forces the FB pin to equal VREF.  The relationship between the output voltage and the divider resistors is given by:

VOUT=1+ RTOPRBOT

Where RTOP is the resistor from VOUT to FB and RBOT is the resistor from FB to ground. The absolute values of these resistors are selected to balance noise immunity (favoured by lower impedance) against quiescent current consumption (favoured by higher impedance). This divider constitutes the sensing element of the outer voltage loop; its accuracy directly influences output voltage regulation.

### Loop Gain, Crossover Frequency, and Stability Margins

The loop gain Ts= Gplants . Hs. Gc(s) describes the propagation of a perturbation around the feedback path and is analysed using Bode plots of magnitude and phase. Three key metrics determine stability and performance:

- Crossover frequency fc : the frequency at which T(fc)=1 0dB. This provides an indication of closed loop bandwidth.

- Phase Margin: the difference between the phase of T(s) at fc and -180o. Values below 45o risk ringing or instability.

- Gain Margin: the amount by which T(s) is below 0 dB at the frequency where phase equals -180o. A margin of at least 10dB is generally desired.

A fundamental limitation in boost converters is the Right-Half-Plane-Zero (RHPZ), which arises because an increase in duty cycle initially reduces the energy delivered to the output before the inductor stores additional energy. Its location is approximately:

fRHPZ= (1-D)2ROUT2πL

Because the RHPZ contributes gain but negative phase shift, the crossover frequency must be placed significantly below fRHPZ (commonly fc ≤ fRHPZ5 ) to preserve stability. This constraint is evaluated at the worst-case operating point.

### Type II Compensation Network

Owing to the single-pole behaviour of the current-mode controlled power stage, a Type II compensator suffices. The network comprises a series resistor Rz and capacitor Cz from the ITH pin to ground, together with a smaller capacitor Cp. The compensator introduces:

- A zero at fZ=12πRZCZ, which provides phase boost near the crossover frequency.

- A high-frequency pole at fP=12πRZCP, which attenuates switching noise.

- An integrator pole at the origin arising from the error amplifier, ensuring zero steady-state error.

Collectively, these elements shape the loop response to achieve the desired crossover frequency with adequate phase margin while maintaining low output impedance and fast transient response.

### Advantages for this Application

The adoption of current-mode control together with a Type II compensator offers several benefits suited to the battery-powered robotic application: simplified compensation requiring only three passive components; excellent line transient rejection through the fast inner current loop; inherent cycle-by-cycle current limiting; and the consistent loop dynamics across the 8-11V input range, as duty cycle remains below 50%. The detailed selection of compensation components and verification of stability margins through AC analysis are presented in Chapter 3.

## Input Protection Topologies

Battery-powered robotic platforms are exposed to a range of input-side fault conditions, including reverse battery connection, undervoltage during deep discharge, and occasional overvoltage transients. Robust protection circuitry is therefore essential to safeguard the LTC1871 controller and downstream electronics while ensuring reliable operation within the specified 8 – 11V battery voltage window. The design employs three complementary protection layers that together form a voltage window restricting active regulation to the validated operating band of 8 – 11.1V. These mechanisms operate independently of the main power stage and do not introduce significant conduction losses under normal conditions.

### Reverse Polarity Protection

Reverse polarity protection is implemented with a P-channel MOSFET placed in series with the positive battery terminal. When the battery is connected with correct polarity, the gate-source voltage of the PMOS is negative, turning the device fully on and presenting a very low on-state voltage drop (typically tens of millivolts at 2A). If the battery is reversed, the gate-source voltage becomes positive, turning the MOSFET off and blocking current flow completely. Compared with a conventional series diode, the P-channel MOSFET solution offers substantially lower conduction loss and negligible heat generation, making it particularly suitable for battery-powered applications.

### Undervoltage Lockout (UVLO)

Undervoltage lockout prevents the converter from operating when the input voltage falls below a safe minimum threshold. This is achieved by a resistive divider connected between the input voltage and the RUN/SS pin of the LTC1871. The divider is dimensioned so that the voltage at the RUN/SS pin remains below the controller’s enabled threshold (nominally 1.23V) when the input voltage is too low. The turn-on threshold is determined by the divider ratio:

VIN,UVLO=VRUN,th1RupperRlower

where VRUN, this the RUN/SS enable threshold voltage. This mechanism ensures that the converter remains in a low-power shutdown state during deep battery discharge, thereby protecting both the controller and the load.

### Overvoltage Lockout (OVLO)

Overvoltage lockout protects the system when the input voltage exceeds the upper limit of the safe operating window. The circuit uses a Zener diode in conjunction with an NPN transistor to clamp the RUN/SS pin to ground when the input voltage rises above the desired threshold. When the Zener enters breakdown, it supplies base current to the NPN transistor, causing the transistor to saturate and pull the RUN/SS pin low, thereby disabling the LTC1871. The trip point is approximately the Zener breakdown voltage plus the base-emitter voltage of the NPN, with fine adjustment provided by the surrounding resistor network. This simple clamp approach was selected for its low component count and adequate precision for the application.

### Combined Voltage Window Protection

The reverse-polarity blocking, the UVLO, and OVLO circuits operate together to create a comprehensive voltage window. The converter is enabled only when the input voltage lies within the defined safe range. Outside the window the LTC1871 is held in shutdown, preventing unregulated boost operation or potential damage. This layered protection scheme provides robust fault tolerance while maintaining high efficiency within the normal operating region. The detailed component selection, threshold tuning, and simulation validation of these protection circuits are presented in Chapter 3.

## PCB Design Considerations for Power Converters

Switching DC-DC converters operating at hundreds of kilohertz generate rapid voltage and current transitions that can produce significant parasitic effects if the printed-circuit-board layout is not carefully optimised. Parasitic inductance in high-current paths leads to voltage spikes, increased switching losses, ringing, and electromagnetic interference, while poor grounding can degrade control loop stability and introduce noise into sensitive analogue signals. Consequently, effective PCB layout is an essential aspect of realising the efficiency, transient response, and electromagnetic compatibility predicted by simulation.

The primary layout objective is the minimisation of high didt switching loops. In a boost converter these loops comprise the path formed by the input capacitor, power MOSFET, inductor, and Schottky diode during the on-interval, and the inductor, diode, and output capacitor during the off-interval. These components must be placed in close physical proximity with short, wide traces or copper pours to reduce parasitic inductance. The LTC1871 controller should be located near the power stage to keep gate-drive and current-sense traces short, thereby preserving clean switching waveforms and accurate current-mode control.

A continuous ground plane on at least one layer provides a low impedance return path for switching currents and serves as a reference plane for the control circuitry. Power and signal grounds are ideally joined at a single point near the controller to avoid common-impedance coupling into the feedback divider and compensation network. Input and output ceramic capacitors are placed immediately adjacent to their respective power pins to supply high-frequency currents locally.

Thermal management is addressed through adequate copper area around the power MOSFET and Schottky diode, supplemented by thermal vias where necessary to transfer heat to the opposite layer. These layout principles ensure that conduction and switching losses are minimised and that junction temperatures remain within safe limits at the rated 2A continuous output current.

The two-layer PCB implemented in this project (60.5mm ×42.5 mm) was designed in accordance with the foregoing considerations. Detailed component placement, routing strategy, and verification of the final layout are presented in Chapter 3.

# Design Methodology

Before the analytical design presented in this chapter, an initial topology discovery phase was

conducted. A SEPIC converter was first considered due to its ability to step up or step down the input voltage. However, persistent LTspice convergence failures (stiff differential equations from the coupled two-inductor network) and the realisation that the 3S Li-Po’s voltage range (8 – 11V) lies strictly below the required 12V output rendered the SEPIC’s buck-boost capability unnecessary. After iterative simulation trials, the topology was pivoted to a simple boost converter using the LTC1871 controller. This discovery process confirmed that a boost topology is topologically sufficient, simplifies the power stage (one inductor, no coupling capacitor), and avoids slope-compensation complexity. The following design specifications and component selections are therefore based on the validated boost converter architecture.

## Design Specifications and Methodology

The converter was designed to meet the electrical specifications derived from the robotic subsystem requirements (Section 1.2). The final design targets are restated in Table 3.1 for convenience.

Table 1: Final design specifications

| Parameter | Value |
| --- | --- |
| Input voltage range (operating) | 8 V to 11 V (3S LiPo) |
| Undervoltage lockout (UVLO) turn-on | 8.0 V (nominal, simulated) |
| Overvoltage lockout (OVLO) trip | 11.1 V (nominal, simulated) |
| Regulated output voltage | 12 V ± 1 % |
| Continuous output current | 2.0 A |
| Transient output current (load step) | 0.5 A → 2.0 A (motor inrush simulation) |
| Switching frequency | 350 kHz ± 10 % |
| Conduction mode | Continuous (CCM) over full load range |
| Output voltage ripple | < 240 mV peak-to-peak (2 %) |
| Efficiency (full load, target) | > 85 % |
| Load regulation (target) | < 1 % |
| Line regulation (target) | < 0.1 % / V |
| Operating temperature range | –40 °C to +85 °C (LTC1871E grade) |

Notes: UVLO and OVLO thresholds are implemented via the resistive divider on the RUN/SS pin and the Zener-NPN clamp circuit, respectively. Actual simulated thresholds (matching the validated LTspice model) are presented in Table 3.4. The transient load step of 2A (from 0.5 A idle) simulates worst-case motor inrush; component ratings provide margin for a conservative 3 A surge.

The design flow followed the three-phase methodology outlined in Section 1.2. First, analytical calculations based on the theoretical relationships derived in Chapter 2 were performed to size all power stage components and the compensation network. Second, the design was implemented in LTspice (Chapter 4) for simulation verification; component values were refined iteratively where simulation revealed parasitic effects not captured by first-order equations. Third, after simulation validation, the schematic was captured in KiCad, and the PCB layout was completed. The final hardware prototype (Chapter 6) was assembled and tested.

## Power Stage Component Selection

All calculation is this section were performed at the worst-case operating point (minimum input voltage Vin=8 V and maximum output current IOUT=2.0 A), which yields the highest duty cycle, peak inductor current, and output capacitor current RMS current. The equations used are those derived in Sections 2.2 and 2.4. The detailed component sizing, stress analysis, and loss calculations are presented in the following sub-sections.

### Switching Frequency and Duty-Cycle Range

The switching frequency was set to fsw=350 kHz using a resistor RRT=68.1 kΩ at the FREQ pin (analytical design, Section 4.3). The corresponding duty cycle range, from the boost transfer ratio D=1- VINVOUT, is:

- At Vin = 11 V: D = 1 - 1112 = 0.083 (8.3 %)

- At Vin = 8 V: D = 1 - 812 = 0.333 (33.3 %)

All duty cycles remain below 50%, ensuring that the LTC1871’s internal slope compensation remains inactive (Section 2.2.2).

### Inductor Selection

To satisfy the CCM boundary condition derived in Equation (2.7) with a minimum margin of 2× at light load, while also providing adequate saturation headroom, an inductance of 10μH was selected (Coil craft XAL6060-103MEC). The ripple current, average current, peak current, and RMS current were calculated using Equations (2.3)– (2.7) from Section 2.2.3:

- Ripple current:
ΔIL=VINDfswL=8×0.333350×103×10×10-6=0.762 A

- Average inductor current:
IL,AVG=IOUT1-D=2.00.667=3.00 A

- Peak inductor current (analytical):
IL,=IL, AVG+ΔIL2=3.00+0.381=3.381 A

- RMS inductor current:
IL,RMS=IL,AVG2+ΔIL212=9.00+0.0484=3.008 A

The inductor’s saturation current rating is 11.2 A, providing a margin of 3.3× over the analytical peak current. The CCM condition was verified at both full load and idle load using Equation (2.7):

- At Iout=2.0 A: Lmin=1.27 μH → 10 µH provides 7.9× margin.

- At Iout=0.5 A: Lmin=5.08 μH → 10 µH provides 2.0× margin, guaranteeing CCM even under light load.

### Power MOSFET Selection

The switching element is an N-channel power MOSFET (IRF7811). Its key parameters are:

- VDSS=30 V (safe for Vout=12 V plus ringing)

- RDS(on)=12 mΩ (typical) at VGS=4.5 V

- Total gate charge Qg≈14 nC

The conduction loss is:
Pcond=IL,RMS2⋅RDS(on)⋅Dmax=9.05×0.012×0.333≈0.036 W

Switching loss (estimated from datasheet formula):
Psw≈12VoutIout(trise+tfall)fsw≈12×12×2.0×40×10-9×350×103≈0.17 W

Total MOSFET loss is approximately 0.21 W, resulting in a modest temperature rise. The SENSE pin of the LTC1871 monitors the drain-source voltage during the on-time, using RDS(on) as a sensing element. No external sense resistor is required, maximising efficiency. The temperature coefficient of RDS(on) (which increases with junction temperature) was considered, but the low duty cycle and moderate current keep the sense voltage within the controller’s range without extra compensation.

### Schottky Diode Selection

The output diode is an MBR735 Schottky rectifier with a 35V repetitive peak reverse voltage rating, 7.5 A average forward current rating, and forward voltage drop of 0.45 V at 2 A. Reverse recovery time is negligible.

The conduction loss in the diode is:
PD1=Iout⋅VF⋅(1-Dmax)=2.0×0.45×0.667≈0.60 W

This is the dominant loss in the power stage, accounting for approximately 75 % of total losses.

### Input and Output Capacitor Selection

#### Output Capacitor Bank

The output capacitor bank must satisfy ripple voltage, RMS current, and transient energy requirements. A hybrid approach was adopted using ceramic MLCC’s for high-frequency ripple filtering and low ESR, plus an electrolytic capacitor for bulk energy storage during load transients.

From Equation (2.12) (Section 2.2.5.1), the minimum ceramic capacitance to meet a ripple specification ΔVout<240 mV is:

Cout,min=IoutDmaxfswΔVout=2.0×0.333350×103×0.24≈7.94 μF

After DC-bias derating (X5R or X7R dielectrics), the effective capacitance of MLCCs can drop to 60–80 % of nominal at 12 V. To provide ample margin, seven 22 µF and two 10 µF (25 V) ceramic capacitors were selected. The derated effective capacitance at 12 V is approximately 77 µF + 12 µF = 89 µF – more than ten times the minimum.

The RMS ripple current that the output capacitor bank must handle (Equation 2.13, Section 2.2.5.3):

IRMS(Cout)=IoutVout-VinVin=2.0×48=1.41 ARMS

Each 22 µF MLCC is rated for ~200 mA RMS at 350 kHz; the parallel combination comfortably exceeds 1.41 A.

An electrolytic capacitor (100 µF, 25 V, ESR ≈ 100 mΩ) was added in parallel to provide bulk energy storage for the 2 A load transient.

#### Input Capacitor

The input capacitor supplies the pulsed current drawn by the MOSFET during the on-time. The RMS ripple current through the input capacitor is:

IRMS(Cin)=ΔIL12=0.7623.464≈0.22 ARMS

A 120 µF, 25 V electrolytic capacitor (C1 in the schematic) provides sufficient filtering, augmented by a 22 µF MLCC directly at the LTC1871 VIN pin for high-frequency decoupling. The resulting input voltage ripple is negligible (< 3 mV), ensuring clean current sensing and stable UVLO operation.

Table 2: Power Stage Component Summary

| Component | Part Number | Key Ratings | Qty |
| --- | --- | --- | --- |
| Inductor | XAL6060-103MEC | 10 µH, 11.2 A sat, 8.3 A RMS | 1 |
| MOSFET | IRF7811 | 30 V, 12 mΩ, 14 nC | 1 |
| Diode | MBR735 | 35 V, 7.5 A, Schottky | 1 |
| Output MLCCs (22 µF) | GRM31CR61E226M | 25 V, X5R, 1206 | 7 |
| Output MLCCs (10 µF) | GRM21BR61E106K | 25 V, X5R, 0805 | 2 |
| Output electrolytic | Generic | 100 µF, 25 V, ESR ≈ 100 mΩ | 1 |
| Input electrolytic | Generic | 120 µF, 25 V | 1 |

Figure 3.1 – Complete power stage schematic

## Feedback Divider and Loop Compensation Network Design

### Output Voltage Feedback Divider

The LTC1871 regulates the FB pin to an internal reference of  1.230 V. Using Equation (2.8), the divider resistors were chosen as RBOT=56 kΩ and RTOP=499 kΩ (1 % tolerance). The resulting output voltage is:

VOUT=1.230149956=1.230×9.911=12.19 V

The output voltage is slightly above 12 V to account for minor resistive drops in the power path. The divider impedance is approximately RTOP∥RBOT≈556kΩ, which limits quiescent current to 12 V / (556 kΩ) ≈ 22 µA while maintaining acceptable noise immunity.

### Type II Compensation Network Design

The compensation network connecting to the ITH pin consists of RZ=8.66 kΩ, CZ=6.8 nF, and CP=100 pF. These values were calculated using the Type II zero and pole equations (Section 2.4.3):

- Zero frequency: fZ=12πRZCZ=12π⋅8.66×103⋅6.8×10-9≈2.70 kHz

- High-frequency pole: fP=12πRZCP=12π⋅8.66×103⋅100×10-12≈184 kHz

The target crossover frequency fc was set to 4.91 kHz, which is approximately one-fifth of the worst-case Right-Half-Plane Zero frequency. The worst-case RHPZ occurs at minimum Vin=8 V (D=0.333) and maximum Iout=2.0 A:

fRHPZ=1-0.333)2⋅122π⋅10×10-6⋅2.0=0.444⋅126.283⋅20×10-6=5.33125.7×10-6≈42.4 kHz

Thus, fc≤fRHPZ/5≈8.5 kHz; the chosen 4.91 kHz satisfies this constraint with margin. The phase margin of the compensated loop was estimated using the method of K factor and found to exceed 60°, with gain margin greater than 15 dB.

Table 3: Compensation Network Parameters.

| Component | Value | Calculated Frequency |
| --- | --- | --- |
| R_Z | 8.66 kΩ | — |
| C_Z | 6.8 nF | 2.70 kHz |
| C_P | 100 pF | 184 kHz |

Figure 3.2 – Compensation network (ITH pin connections)

### Verification of Expected Loop Performance

The design values were validated through simulated AC analysis (Section 4.7), which confirmed a crossover frequency of 4.91 kHz, phase margin of 55.5°, and gain margin of 19.1 dB. These results meet the stability criteria established in Section 2.4.2 and ensure robust operation under all specified input and load conditions.

## Input Protection Circuit Implementation

### Reverse Polarity Protection

A P-channel MOSFET (AO6407, later replaced with FDS4435A in simulation) was placed in series with the input. Its gate is connected to ground via a resistor, and its source to the battery positive terminal. Under normal polarity, the gate-source voltage is negative (gate at 0 V, source at battery voltage), turning the FET on with very low resistance (RDS(on)≈50 mΩ). Reverse battery connection forces the gate positive relative to the source, turning the FET off and blocking current. This scheme dissipates far less power than a series Schottky diode.

### Undervoltage Lockout (UVLO)

To achieve a turn-on threshold of 8 V (as validated in simulation), a resistive divider with R6=49 kΩ and R7=10 kΩ (1 %) was selected. The effective threshold voltage required from the RUN pin to turn on at 8 V is:

VRUN,th(eff)=8×R7R6+R7=8×1059=1.356 V

This value lies within the datasheet’s maximum specification (1.35 V) and accounts for the slightly higher threshold observed in the LTspice model. The nominal turn-off voltage is approximately 7.9 V due to internal hysteresis (typical 100 mV). The lower-impedance divider (49 kΩ, 10 kΩ) improves noise immunity compared to a higher-impedance alternative.

### Overvoltage Lockout (OVLO)

An overvoltage protection circuit using a BZX84C10 Zener diode (10 V breakdown) and MMBT3904 NPN transistor was implemented to prevent operation above approximately 11.1 V. When the input voltage rises such that the Zener conducts and the transistor base-emitter voltage reaches ≈ 0.6 V, the transistor pulls the RUN pin low, disabling the controller. The trip point is:

VIN, OVLO=VZ+VBE=10+0.6=10.6 V

A series resistor (R5) was adjusted to set the Zener current. The final simulated trip point was 11.1 V due to the transistor’s finite current gain and resistor loading, which was deemed acceptable because the Li-Po nominal maximum is 12.6 V, and the protection still activates before dangerous levels.

### Combined Voltage Window Protection

The three protection layers – reverse polarity (MOSFET), UVLO (resistive divider), and OVLO (Zener + transistor) – operate independently of the power stage and do not interfere with normal boost operation. The operating window of approximately 8 V to 11.1 V ensures the converter operates only within a safe range of the 3S Li-Po battery.

Table 4.4: Protection thresholds: theoretical vs. simulated

| Protection | Theoretical Threshold | Simulated Threshold |
| --- | --- | --- |
| UVLO turn-on | 8.0 V (designed) | 8.0 V |
| UVLO turn-off | ~7.9 V (hysteresis) | 7.9 V |
| OVLO trip | 10.6 V (initial) | 11.1 V (final) |

Figure 3.3 – Input protection sub-circuit (Insert schematic showing the P-channel FET, UVLO divider, and OVLO Zener diode + transistor.)

## Schematic Capture, PCB Layout, and Prototype Realisation

### Schematic Capture in KiCad

The LTspice validated schematic (Figure 3.1) was translated into KiCad (version 7.0) for PCB design. All components were assigned realistic footprints (0402/0805/1206 for passives, MSOP-10 for the LTC1871, etc.). Electrical Rule Check (ERC) was performed and all warnings resolved. Netlist consistency was verified by comparing component counts and connections between the simulation and layout schematics.

### PCB Layout Strategy

The principles set out in Section 2.6 (PCB Layout Considerations) guided the PCB layout:

- High di/dt loops minimised: The power loop (MOSFET, diode, output capacitor) was made as compact as possible, with trace widths of at least 2 mm for 2 A continuous current (1 oz copper, 40 °C rise: ~1 mm width sufficient, but 2 mm used for safety).

- Ground plane: A continuous ground plane was used on the bottom layer (2-layer board) with multiple vias stitching the top-side ground pads.

- Thermal management: The SI4894BDY has a ground pad; vias were placed under it to conduct heat to the bottom ground plane. The MBR735 was mounted with a copper pour on top for heat spreading.

- Component placement: The LTC1871 was placed close to the MOSFET gate, and the SENSE pin trace was kept short and direct. The feedback divider (R2, R3) was placed near the FB pin with a Kelvin connection to the output capacitor positive terminal.

- Trace width calculation: For 2 A, using IPC-2221, a trace width of 1 mm (≈40 mil) on 1 oz copper gives a temperature rise of about 10 °C. Critical traces (input supply, MOSFET drain, diode cathode) were made 2 mm wide.

The board dimensions are 60.5 mm × 42.5 mm. All components are on the top layer; the bottom layer is primarily a ground plane with a few routing traces for low-frequency signals.

Figure 3.4 – PCB layout: (a) top layer, (b) bottom layer – board size 60.5 mm × 42.5 mm

Table 5 - Bill of Materials

| Ref | Description | Part Number | Footprint | Qty | Total (€) |
| --- | --- | --- | --- | --- | --- |
| U1 | LTC1871 controller, MSOP-10 | LTC1871EMS#PBF | MSOP-10 | 1 | 3.80 |
| Q1 | IRF7811 MOSFET, SO-8 | IRF7811TRPBF | SO-8 | 1 | 1.20 |
| D1 | MBR735 Schottky diode | MBR735 | TO-220 | 1 | 0.45 |
| L1 | 10 µH inductor | XAL6060-103MEC | 10×10 mm SMD | 1 | 2.80 |
| C1a (×7) | 22 µF, 25 V, X5R MLCC | GRM31CR61E226M | 1206 | 7 | 3.15 |
| C1b (×2) | 10 µF, 25 V, X5R MLCC | GRM21BR61E106K | 0805 | 2 | 0.50 |
| C5 | 100 µF, 25 V electrolytic | Generic | Radial 8 mm | 1 | 0.20 |
| C_in | 120 µF, 25 V electrolytic | Generic | Radial 8 mm | 1 | 0.25 |
| R2 | 56 kΩ, 1 % (R_BOT) | — | 0805 | 1 | 0.02 |
| R3 | 499 kΩ, 1 % (R_TOP) | — | 0805 | 1 | 0.02 |
| R_Z | 8.66 kΩ, 1 % | — | 0805 | 1 | 0.02 |
| C_Z | 6.8 nF, 50 V | — | 0805 | 1 | 0.03 |
| C_P | 100 pF, 50 V | — | 0805 | 1 | 0.02 |
| R_RT | 68.1 kΩ, 1 % | — | 0805 | 1 | 0.02 |
| R6 | 49 kΩ, 1 % (UVLO top) | — | 0805 | 1 | 0.02 |
| R7 | 10 kΩ, 1 % (UVLO bottom) | — | 0805 | 1 | 0.02 |
| R5 | 1 kΩ (OVLO series) | — | 0805 | 1 | 0.02 |
| D2 | BZX84C10 Zener | BZX84C10 | SOT-23 | 1 | 0.08 |
| Q2 | BC547 NPN | BC547 | SOT-23 | 1 | 0.04 |
| D3 | AO6407 (reverse protection) | AO6407 | SOT-23-6 | 1 | 0.40 |
| Total |  |  |  |  | 11.30 |

### Fabrication and Assembly

Gerber files were generated from KiCad and sent to JLCPCB for fabrication (2-layer, 1 oz copper, ENIG finish). The board turnaround was approximately one week. Assembly was performed manually in the university laboratory using a soldering iron and tweezers. The MSOP-10 package required careful alignment; a fine-tip soldering iron and flux were used. After assembly, the board was visually inspected under a microscope and tested for shorts before power-up.

Figure 3.5 – Assembled prototype (Insert photograph of the finished PCB.)

## Summary of Design Decisions

The design methodology presented in this chapter translated the theoretical relationships from Chapter 2 into a complete set of component values and a physical PCB. Key decisions were:

- Selection of a boost topology (instead of SEPIC) because VIN always lies below VOUT.

- Use of a 350 kHz switching frequency to balance efficiency and inductor size.

- A 10 µH inductor with 3.3× saturation margin over the analytical peak current, ensuring CCM at all loads.

- Hybrid output capacitor bank (MLCCs + electrolytic) to meet ripple and transient requirements.

- Type II compensation network sized to place the crossover well below the RHPZ, guaranteeing stability.

- Input protection with UVLO (8 V turn-on using 49 kΩ/10 kΩ divider) and OVLO (~11.1 V) to prevent operation outside the Li-Po safe range, with low-impedance resistors for noise immunity.

The completed design was then subjected to the simulation campaign described in Chapter 4, where all performance metrics were verified.

# Simulation Methodology

## Simulation Environment and Solver Configuration

All simulations were performed using LTspice version 24.1.3 (Analog Devices). The following solver settings were applied to improve convergence for the switching converter:

- . options gmin=1e-12: reduces the minimum conductance, aiding convergence in highly non-linear circuits.

- . options cshunt=1e-15: adds a small shunt capacitance to each node, supressing numerical oscillations.

- Integration method: trapezoidal (default in LTspice), selected for its superior energy conservation in switching circuits.

During initial DC operating point analysis, the Direct Newton iteration failed to converge. This is common for circuits containing behavioural models with steep slopes. Gmin stepping was therefore invoked by LTspice automatically, which gradually increases conductance across semiconductor junctions until a stable operating point is found. The failure of Direct Newton is not indicative of a design flaw. It reflects the complexity of the LTC1871 macro model and the power stage.

The transient simulation duration was set to .tran 8m startup. The startup flag forces the input voltage source to rise from zero over approximately 1ms, simulating a realistic battery connection. The full 8ms duration was chosen because:

- The startup transient settles within 2ms.

- A 4ms to 5ms window captures steady-state behaviour before load step.

- The load step occurs at 5ms, and the converter recovers within 1ms.

- A 6ms to 7ms window provides steady-state post-step measurements, leaving a 1ms safety margin after recovery.

## SPICE Models and Component Representation

All simulation models are documented below to ensure reproducibility.

Table 6: LTspice Component Models and Library Sources

| Component | Model / Library | Source | Notes |
| --- | --- | --- | --- |
| LTC1871 controller | LTC1871.sub (macro model) | Analog Devices [19] | Behavioural model; captures functional behaviour, current-mode control, protection features. May not perfectly predict switching losses or gate drive dynamics. |
| N-channel MOSFET (IRF7811) | standard.mos (NMOS) | LTspice built-in | Level 1 model with parameters extracted from Vishay datasheet. |
| P-channel MOSFET (AO6407 / FDS4435A) | standard.mos (PMOS) | LTspice built-in | Used for reverse polarity protection. |
| Schottky diode (MBR735) | standard.dio | LTspice built-in | Includes forward voltage drop and junction capacitance. |
| Zener diode (BZX84C10VLY) | standard.dio (zeners) | LTspice built-in | 10 V breakdown voltage. |
| NPN transistor (OVLO) | standard.bjt | LTspice built-in | Used MMBT3904 model (corresponds to OVLO circuit in validated netlist). |
| Inductor L1 | behavioural | Ideal with Rser=14m | 10 µH, 14 mΩ series resistance (typical DCR). |
| Capacitors C1a, C1b, C5 | behavioural | Ideal with series resistance as shown | Derated capacitance values (77 µF, 12 µF, 100 µF). |
| Input voltage source V1 | behavioural | {VIN} parameterised | Allows parametric step. startup flag enabled. |
| Load current source I1 | behavioural | PWL function | Described in Section 4.3. |

All passive components (resistors, ceramic capacitors) are modelled as ideal with nominal values and tolerances not simulated. The LTC1871 macro model has known limitations: it does not model internal power consumption of the gate driver with high accuracy, and it assumes ideal slope compensation. However, for the purpose of verifying regulation, ripple, transient response, and stability, it is sufficient.

## Test Circuit Configuration

The final simulation schematic (reproduced in Figure 4.1) implements the boost converter power stage, the LTC1871 controller, and the input protection network as derived in Chapter 3.

Figure 2: LTspice simulation schematic (LTC1871_Validated_Final_Fixed_V2.asc

Key elements are:

- Parameterised input voltage: “V1” with “{VIN}”. The directive “.param VIN=9.6” sets a nominal value; a “.step param VIN list 7.5 8.0 9.6 11.0 11.1 12.0” is used for parametric sweeps (Section 4.4).

- Pulsed load current: “I1” is defined as “PWL (0 0.5 0.005 0.5 0.0051 2 0.008 2)”.
Interpretation: 0.5 A from t=0 to 5 ms, then a linear ramp to 2 A over 100 µs (starting at 5 ms), and finally constant 2 A from 5.1 ms to 8 ms. The 100 µs ramp approximates the inrush current of a DC motor during startup, which appears as a fast load change to the converter.

- Input protection sub-circuit: Includes the P-channel MOSFET M2, UVLO divider (R6 = 49 kΩ, R7 = 10 kΩ), and OVLO Zener D3 + NPN transistor Q1(MMBT 3904).

All measurement directives ( .meas) are defined in the schematic and summarised in Section 4.5.

## Parametric Sweep Strategy

To validate the converter’s behaviour across the entire expected input voltage range and beyond, a parametric step sweep was configured, using the SPICE e command:

“.step param VIN list 7.5 8.0 9.6 11.0 11.1 12.0”

Each test point corresponds to a distinct operating region of the input protection scheme:

Table 7: Input Voltage Sweep Test Points for Protection Circuit Verification

| Test Point (VIN) | Region | Expected State |
| --- | --- | --- |
| 7.5 V | Below UVLO | Converter off (RUN pin < threshold) |
| 8.0 V | UVLO lower boundary | Converter should start regulating |
| 9.6 V | Nominal operating point | Normal boost regulation |
| 11.0 V | Near upper boundary | Normal regulation (OVLO not yet tripped) |
| 11.1 V | OVLO upper boundary | Converter should still regulate (trip point is above 11.1 V) |
| 12.0 V | Above OVLO | Converter should shut down |

The six points were simulated sequentially in a single run. The results (presented in Chapter 5) confirm the protection thresholds and verify regulation within the 8.0-11.1V window.

## Measurement Definitions

All key performance metrics were extracted using LTspice’s .meas directives. Table 8 lists each metric, the exact directive, the measurement window, and the rationale for window selection. These definitions ensure that any engineer with the same netlist can reproduce the numbers exactly.

Table 8: Measurement Directives and Windows

| Metric | .meas Directive | Window | Rationale |
| --- | --- | --- | --- |
| Pre-step output voltage | AVG V(OUT) FROM 4m TO 5m | 4–5 ms | After startup settles (by 2 ms), before load step (5 ms). |
| Post-step output voltage | AVG V(OUT) FROM 6m TO 7m | 6–7 ms | After load step transient has fully recovered. |
| Load transient droop (voltage) | MIN V(OUT) FROM 5.1m TO 5.4m | 5.1–5.4 ms | Captures minimum value during the 100 µs ramp and subsequent decay. |
| Droop in millivolts | PARAM (Vout_before − Vout_droop)*1000 | Derived | Calculated from captured pre-step average and minimum. |
| Droop as percentage | PARAM (Vout_before − Vout_droop)/Vout_before*100 | Derived | Expresses droop relative to nominal output. |
| Output ripple (peak-peak) | PP V(OUT) FROM 6m TO 7m | 6–7 ms | Steady-state at full load, after recovery. |
| Ripple in millivolts | PARAM Vout_ripple*1000 | Derived | Converts peak-peak ripple to millivolts for readability. |
| Peak inductor current | MAX I(L1) FROM 4m TO 5m | 4–5 ms | Pre-step, light load (0.5 A) – used for saturation check. |
| Recovery time | FIND TIME WHEN V(OUT)=12.15 CROSS=2 TD=5m | After 5 ms | Time at which output returns to 12.15 V (≈99% of target) after the step. |
| Input power | AVG V(IN)*(−I(V1)) FROM 6m TO 7m | 6–7 ms | Post-step, full load, steady-state. |
| Output power | AVG V(OUT)*I(I1) FROM 6m TO 7m | 6–7 ms | Post-step, full load, steady-state. |
| Efficiency | PARAM Pout/Pin*100 | Derived | Calculated after both power measurements. |

In preliminary simulation runs, a shorter total time (5 ms) was used with a load step at 2 ms and post-step window at 3–4 ms. This left insufficient settling time after the startup transient. The final simulation uses 8 ms total time, a load step at 5 ms, and measurement windows starting at 4 ms and 6 ms. This guarantees at least 2 ms of steady-state before each measurement, eliminating transient artefacts.

## Loop Stability Verification (Analytical)

Loop stability was verified analytically using a MATLAB model of the control loop, rather than through an LTspice AC sweep. The LTC1871 implements current-mode control, which simplifies the power stage transfer function to approximately a single pole. The control-to-output transfer function, combined with the Type II compensator designed in Section 3.3.2, was used to construct the open-loop gain T(s).

The following parameters were entered into the MATLAB script:

- Power stage: L = 10μH, Cout = 89μF (derated MLCCs), Rload = VoutIout = 12V2.0 A = 6Ω, duty cycle D = 0.333 (worst case)

- Compensator: Rz = 8.66kΩ, Cz = 6.8nF, Cp = 100pF.

- Current sensing gain derived from Rds = 12mΩ and the LTC1871’s internal gain factors (datasheet)

The loop gain T(s) was evaluated over a frequency range of 10 Hz to 1 MHz. From the resulting Bode plot (presented in Chapter 5), the following stability margins were extracted:

- Crossover frequency: fc=4.91 kHz

- Phase margin: 55.5∘ (well above the 45∘ minimum)

- Gain margin: 19.1 dB (exceeding the 10 dB guideline)

These margins satisfy the criteria established in Section 2.4.2. The analytical verification confirms that the Type II compensation network provides stable operation under the nominal operating condition (Vin=9.6 V, Iout=2.0 A). A full LTspice AC simulation was not performed due to convergence difficulties with the behavioural micromodel when breaking the feedback loop; this is noted as a potential future improvement in Chapter 6.

Glossary of terms [OPTIONAL]

Your report may not require a glossary of terms, but if it does, place it here.

Nomenclature [OPTIONAL]

Your report may not require a nomenclature section. Normally this is only required if there is extensive use of mathematics throughout the report and the symbols or nomenclature are not obvious. If nomenclature is needed, place it here.

References

List all your references here using a standard consistent style. See handbook for detailed guidelines.

Appendix A [OPTIONAL]

Appendices are optional and may not be required by your project. If you do require appendices, place them here.

Risk Assessment

Include your unsigned risk assessment form here, i.e. no signatures. This is the electronic version of risk assessment form before it was signed by you or the safety officers. The content should match the most recently signed off version except that, for GDPR and future sharing of the report, there are no signatures and no student numbers included.