# LTspice Component Models and Library Sources

| Component | Model / Library | Source | Notes |
| --- | --- | --- | --- |
| LTC1871 controller | LTC1871.sub (macro model) | Analog Devices [19] | Behavioural model; captures functional behaviour, current‑mode control, protection features. May not perfectly predict switching losses or gate drive dynamics. |
| N‑channel MOSFET (IRF7811) | standard.mos (NMOS) | LTspice built‑in | Level 1 model with parameters extracted from Vishay datasheet. |
| P‑channel MOSFET (AO6407 / FDS4435A) | standard.mos (PMOS) | LTspice built‑in | Used for reverse polarity protection. |
| Schottky diode (MBR735) | standard.dio | LTspice built‑in | Includes forward voltage drop and junction capacitance. |
| Zener diode (BZX84C10VLY) | standard.dio (zeners) | LTspice built‑in | 10 V breakdown voltage. |
| NPN transistor (OVLO) | standard.bjt | LTspice built‑in | Used MMBT3904 model (corresponds to OVLO circuit in validated netlist). |
| Inductor L1 | behavioural | Ideal with Rser=14m | 10 µH, 14 mΩ series resistance (typical DCR). |
| Capacitors C1a, C1b, C5 | behavioural | Ideal with series resistance as shown | Derated capacitance values (77 µF, 12 µF, 100 µF). |
| Input voltage source V1 | behavioural | {VIN} parameterised | Allows parametric step. startup flag enabled. |
| Load current source I1 | behavioural | PWL function | Described in Section 4.3. |

Table 4.1 – LTspice Component Models and Library Sources