# LTspice Measurement Definitions

| Metric | .meas Directive | Window(s) | Rationale |
| --- | --- | --- | --- |
| Pre-step output voltage | AVG V(OUT) FROM 4m TO 5m | 4 ms – 5 ms | After startup settles (by 2 ms), before load step (5 ms). |
| Post-step output voltage | AVG V(OUT) FROM 6m TO 7m | 6 ms – 7 ms | After load step transient has fully recovered. |
| Load transient droop (voltage) | MIN V(OUT) FROM 5.1m TO 5.4m | 5.1 ms – 5.4 ms | Captures minimum value during the 100 µs ramp and subsequent decay. |
| Droop in millivolts | PARAM (Vout_before - Vout_droop)*1000 | Derived | Calculated from captured pre-step average and minimum. |
| Droop as percentage | PARAM (Vout_before - Vout_droop)/Vout_before*100 | Derived | Expresses droop relative to nominal output. |
| Output ripple (peak-to-peak) | PP V(OUT) FROM 6m TO 7m | 6 ms – 7 ms | Steady-state at full load, after recovery. |
| Peak inductor current | MAX I(L1) FROM 4m TO 5m | 4 ms – 5 ms | Pre-step, light load (0.5 A) – used for saturation check. |
| Recovery time | FIND TIME WHEN V(OUT)=12.15 CROSS=2 TD=5m | After t=5 ms | Time at which output returns to 12.15 V (≈99 % of target) after the step. |
| Input power | AVG V(IN)*(-I(V1)) FROM 6m TO 7m | 6 ms – 7 ms | Post-step, full load, steady-state. |
| Output power | AVG V(OUT)*I(I1) FROM 6m TO 7m | 6 ms – 7 ms | Post-step, full load, steady-state. |
| Efficiency | PARAM Pout/Pin*100 | Derived | Calculated after both power measurements. |

Table 4.3 – LTspice Measurement Definitions and Analysis Windows