# Protection Circuit Test Points

| Test Point (VIN) | Region | Expected State |
| --- | --- | --- |
| 7.5 V | Below UVLO | Converter off (RUN pin < threshold) |
| 8.0 V | UVLO lower boundary | Converter should start regulating |
| 9.6 V | Nominal operating point | Normal boost regulation |
| 11.0 V | Near upper boundary | Normal regulation (OVLO not yet tripped) |
| 11.1 V | OVLO upper boundary | Converter should still regulate (trip point is above 11.1 V) |
| 12.0 V | Above OVLO | Converter should shut down |

Table 4.2 – Input Voltage Sweep Test Points for Protection Circuit Verification